


                    [image: image1.jpg]



	[image: image2.jpg]


  [image: image3.jpg]


  [image: image4.jpg]



	Content:

1) No. 1 solid Aluminium fuel lid door

2) Fitting instructions

3) Sealing ring

Use: one 3.5 mm allen key and one 8 mm spanner 


	 [image: image5.jpg]


       [image: image6.jpg]



	[image: image7.jpg]




	1 - At first take the back left wheel away.
	2 - Then take the wheel cover away too.

	[image: image8.jpg]



	[image: image9.jpg]




	3 - Now take the fuel lid away.
	4 - From underneath, take the fuel pipe away.

	[image: image10.jpg]



	[image: image11.jpg]




	5 – After opening the fuel lid door, eliminate the whole upper rubber part of the fuel pipe and subsequently the door.
	6 - Wet the sealing ring by a spray or some vaseline grease.

	[image: image12.jpg]



	[image: image13.jpg]




	7 - Insert the sealing ring on the top of the fuel pipe.
	8 - Place the sealing ring on the top of the fuel pipe, before fitting it onto the car body by tightening the inferior screw.

	[image: image14.jpg]



	[image: image15.jpg]




	9 -Before placing the fuel pipe in its place, make a 4 or 5 mm diameter hole on the car body, to let water (rain, etc.) flow down.
	10 - Now place the chromed fuel lid door.

At first, on the right side fit the bracket of the lid door frame.

	[image: image16.jpg]



	[image: image17.jpg]




	11 - Then, on the left side, put the black plastics supports on the car body: outside move the two bolts of the supports by the 3.5 mm allen key and inside by the 8 mm spanner.
	12 - Fitting is completed.

 











Fitting instructions











Solid aluminium fuel lid door














 New Mini








       BMW








1
2

