	[image: image13.jpg]



	Fitting instructions


	Jeep Cherokee

	
	Real wood interior kit

Part no.: KJ-EHS-01
	


[image: image1.png]STARTECH °



[image: image6.wmf] 

[image: image2.jpg]



	[image: image7.jpg]


                                                                    

                                                                     [image: image3.jpg]454 -
LOCTITE




[image: image8.png]3M
8984



                                                                    [image: image4.jpg]



[image: image9.jpg]


                                                                      
	Content:

1) No. 6 real wood parts:

· 1 central console;

· 1 gear base panel;

· 2 (left and right) electrical mirror cover switches;

· 1 front electrical windows opener panel;

· 1 rear electrical windows opener panel;

2) No. 1 tube of UHU GREENIT and 1 tube of LOCTITE

3) No. 1 sheet of “felt” (black adhesive stripes)

4) Fitting instructions

 

	[image: image5.png]STARTECH °




	Fitting instructions


	Jeep Cherokee

	
	Real wood interior kit

Part no.: KJ-EHS-01
	

	[image: image10.jpg]



	Thoroughly clean the surface of the fitting areas on all its surface by means of suggested 3M 8984 or similar degreasing agents (industrial cleaners).

If you see that the aluminium parts of the car produce undesired reflections from below the  wood parts, apply  little stripes of “felt” (black adhesive) on the edge of the car part.

	[image: image11.jpg]




	Place the single part over the proper area in its correct position. 

It is suggested to hold the part with fingertips on the edges and to check its coincidence with the original surface.

After checking the correct position, apply the UHU GREENIT on the backside of each wood part and also on the corresponding surface of the car, until a visible layer of adhesive can be seen. 

Check that the contact glue is dry enough and, after 10-15 minutes at a 20° C temperature, put small drops of the instant glue LOCTITE on the  fitting area, over the UHU GREENIT layer. Drops must be applied about 3-5 cm from one to another, and not too close to the edges in order to avoid overflow.
This will grant the immediate adhesion of the wooden part.

Parts must be fitted with a vigorous pressure on the surface with both hands, trying to push with uniformity on the surface.

Then the fitting procedure is completed.

Definitive drying of glue will end after 36-48 hours, consequently it is suggested not to pull the part backwards within this term.



	[image: image12.wmf] 


	


� EMBED Word.Picture.8  ���





6.























1
2

_1128497637.doc
[image: image1.png]






